Quiz 6 solution			 		60 minutes

1.  (35)  For the following state table, find the equations for the output and the inputs to D flip flops for each of the following state assignments.  DO NOT draw block diagrams!  (Note that the three solutions are quite different from each other.)  See worksheet on next page.

|  | q | q | * | z |
| --- | --- | --- | --- | --- |
|  |  | x = 0 | x = 1 |  |
|  | A | C | B | 0 |
|  | B | D | A | 1 |
|  | C | A | B | 1 |
|  | D | B | A | 0 |

a)				   b)					c)

| q | q1 | q2 |  | q | q1 | q2 |  | q | q1 | q2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 0 |  | A | 0 | 0 |  | A | 0 | 0 |
| B | 0 | 1 |  | B | 0 | 1 |  | B | 1 | 1 |
| C | 1 | 0 |  | C | 1 | 1 |  | C | 1 | 0 |
| D | 1 | 1 |  | D | 1 | 0 |  | D | 0 | 1 |

|  | x | q1 | q2 | q1* q2* |  | x | q1 | q2 | q1* q2* |  | x | q1 | q2 | q1* q2* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 0 | 0 | 1 0 | A | 0 | 0 | 0 | 1 1 | A | 0 | 0 | 0 | 1 0 |
| B | 0 | 0 | 1 | 1 1 | B | 0 | 0 | 1 | 1 0 | D | 0 | 0 | 1 | 1 1 |
| C | 0 | 1 | 0 | 0 0 | D | 0 | 1 | 0 | 0 1 | C | 0 | 1 | 0 | 0 0 |
| D | 0 | 1 | 1 | 0 1 | C | 0 | 1 | 1 | 0 0 | B | 0 | 1 | 1 | 0 1 |
| A | 1 | 0 | 0 | 0 1 | A | 1 | 0 | 0 | 0 1 | A | 1 | 0 | 0 | 1 1 |
| B | 1 | 0 | 1 | 0 0 | B | 1 | 0 | 1 | 0 0 | D | 1 | 0 | 1 | 0 0 |
| C | 1 | 1 | 0 | 0 1 | D | 1 | 1 | 0 | 0 0 | C | 1 | 1 | 0 | 1 1 |
| D | 1 | 1 | 1 | 0 0 | C | 1 | 1 | 1 | 0 1 | B | 1 | 1 | 1 | 0 0 |

a.	z  =  q1' q2  +  q1q2'

D1  =  x' q1'

D2  =  x' q2  + x q2'

7/14 + x' NOT

b.	z  =  q2

D1  =  x' q1'

D2  =  x' q2'  + q1' q2'

+  x q1 q2

5/			5/12 + x' NOT

c.	z  =  q1

D1  =  x' q1'  +  x q2'

D2  =  x' q2  +  x q2'

5/10 + x' NOT

2.  (35)  Design a counter that goes through the sequence

0  9  2  11  4  13  6  15  8  1  10  3  12  5  and repeat

Where the four flip flops are labelled A, B, C, D and (and, for example, A=0, B=1, C= 0, D=1 represents 5.)  A is a JK flip flop, B is an SR flip flop, C is a D flip flop and D is a T flip flop.  Show the input equations and a block diagram.  (Some blank maps are given on the next page.)

|  | A | B | C | D | A* | B* | C* | D* |  | SB | RB |  | TD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | 0 | X |  | 1 |  |
|  | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |  | 0 | X |  | 1 |  |
|  | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |  | 0 | X |  | 1 |  |
|  | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |  | 1 | 0 |  | 1 |  |
|  | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | X | 0 |  | 1 |  |
|  | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |  | 0 | 1 |  | 1 |  |
|  | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | X | 0 |  | 1 |  |
|  | 0 | 1 | 1 | 1 | X | X | X | X |  | X | X |  | X |  |
|  | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 0 | X |  | 1 |  |
|  | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  | 0 | X |  | 1 |  |
|  | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |  | 0 | X |  | 1 |  |
|  | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |  | 1 | 0 |  | 1 |  |
|  | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | X | 0 |  | 1 |  |
|  | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | X | 0 |  | 1 |  |
|  | 1 | 1 | 1 | 0 | X | X | X | X |  | X | X |  | X |  |
|  | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |  | 0 | 1 |  | 1 |  |

BONUS (5 points)  Show a state diagram, indicating what happens if this starts in one of the unused states (7 or 14).  Be sure to indicate which of the multiple solutions you use for R.

A*

0111		1110

JA  =  B'  +  D'			   O		   1

KA  =  B'  +  C'			   0		   0

SB					RB

SB  =  B' C D							   	   0	          0

RB  =  {A' C' D  or  A' B D}  +  {A B C  or  B C D}		  1/0		1/0

C*

DC  =  C D'  +  B' C' D  +  A B D'   					      0		   1

TD  =  1

Thus, 0111 goes either to 0000 or 0100 (depending on RB).

1110 goes wither to 1011 or 1111 (depending on RB).

3.  Show the state table or the state diagram for a Mealy system with one input, x, and one output, z, such that z = 1 iff there has been a pattern of 1 0 1  or of

0 1 0 under each of these assumptions:

a)  (20)  Overlapping is allowed (4 states)

b)  (10)  Overlapping is not allowed (5 states)

Example

x	0 0 1 0 1 0 1 1 0 1 0 1 1 0 0 1 0

za	? 0 0 1 1 1 1 0 0 1 1 1 0 0 0 0 1

zb	? 0 0 1 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0