Procedure: Rapid Infectious Mononucleosis Test

Procedure Goal: To determine the presence of antibodies associated with infectious mononucleosis using whole blood, serum or plasma

OSHA Guidelines:

Materials: Infectious mononucleosis test kit, patient blood sample, and a watch or timer

Method:

- Review the laboratory request form and gather the necessary supplies.

- Greet the patient, confirm the patient’s identity and introduce yourself.

- Explain the procedure.

- Wash your hands and put on required PPE.

- Obtain a sample of the patient’s blood using appropriate venipuncture technique.

- Process the blood sample to obtain whole blood, plasma or serum as required by the testing procedure.

- Open the test kit and check the expiration date.

- Run the recommended controls.

Rationale

To ensure the test is working correctly.

- Place the required amount of blood, serum, or plasma following manufacturer’s instructions onto the testing device.

- Add testing reagent or reagents according to manufacturer’s instructions to the testing device.

- Wait the required amount of time. Do not go over the recommended time.

Rationale

Reading the test too early can result in a false negative and reading the test too long after the recommended time could result in a false positive.

- Read the results and record them in the patient’s chart.

- Discard the testing supplies according to OSHA regulations.

- Disinfect the work area.

- Remove your PPE and wash your hands.