Procedure: Performing Sanitization with an Ultrasonic Cleaner

Procedure Goal: To decontaminate items safely and effectively using a ultrasonic cleaner

OSHA Guidelines:

Materials: Ultrasonic cleaner, contaminated items and instruments, ultrasonic cleaning fluid, manufacturer's directions

Method:

- Review the manufacturer's directions for safe operation of the ultrasonic cleaner.

- Fill the container of the ultrasonic cleaner with water. Look for the fill line on the machine.  In some causes you may use distilled water.

- Add the directed amount of ultrasonic cleaning fluid. Typically only a small amount of fluid is used. Check the directions.

- Plug in and turn on the ultrasonic cleaner.  Some cleaners require a warm up period. Check the instructions.

- Separate instruments and equipment made of different metals

Rationale: Different metals may fuse together during the cleaning process.

- Separate instruments with sharp points to avoid injury.

- Open hinges on instruments and equipment.

Rationale: Contaminated materials can become trapped between two surfaces.

- Place instruments and equipment in the ultrasonic cleaner but do not overfill.

- Close the lid, turn on the machine or timer, and wait for the cycle to be completed.

- Rinse each instrument or piece of equipment in cool running water and then distilled or demineralized water as policy dictates.

Rationale: Ultrasonic cleaning fluid may cause damage to instruments or equipment.

- Dry each instrument or piece of equipment.

- Prepare each item for storage or further disinfection or sterilization.