Quiz 4				40 minutes

1.  Consider the three functions shown below.  Find all of the minimum sum of products for each.  Treat each as a separate problem, but indicate what sharing (of prime implicants) can be done.

2.  Find the equations for a minimum two-level NAND gate circuit.  All you need to show is the maps and the sum of product equations, indicating the sharing.  This solution requires 10 gates/32 gate inputs.  (The comparable solution to problem 1 requires 14 gates/42 gate inputs.)

3.  Implement the functions of problems 1 and 2 on the PAL shown below.  Be sure to label all inputs and outputs.

4.  Implement the functions of problems 1, 2, and 3 on the PLA shown below.  Full credit if you use 9   or fewer terms.  Be sure to label all inputs and outputs.  A copy of the map is shown for your convenience.

5.  Consider the following two functions of 5 variables.

F (A, B, C, D, E)  =  ∑ m (0, 1, 2, 3, 4, 5, 6, 7, 19, 22)

G (A, B, C, D, E)  =  ∑ m (1, 2, 7, 24, 25, 26, 27, 28, 29, 30, 31)

Implement them using only the ROM shown on the next page.

6.  Implement the functions of problem 5  using only 3-input, 8-output, active low decoders, each with one active low enable input and two NAND gates (with as many inputs as are necessary).  A truth table and a block diagram of the decoder is shown below.  BONUS if you do it with only two 4-input NAND gates.

| EN' | A1 | A2 | A3 | Y0 | Y1 | Y2 | Y3 | Y4 | Y5 | Y6 | Y7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X | X | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

5.