Midterm Solution                       1 Hour, 50 minutes

Do any 10 of the 12 problems.  Make sure it is clear which one you do not want graded (or I will grade the first 10 problems).  All problems are worth 20 points.

1.  The following function is already in minimum sum of products form.  Show the equation and a block diagram for an implementation using only 2-input NAND gates.  No gate may be used as a NOT.  All variables are available both uncomplemented and complemented.  For full credit, find a solution using 14 gates; partial credit for more gates or one or two  3-input gates.  Bonus of up to 5 points for less.

F  =  A' B' C' D'  +  B' C D' E  +  B D  +  B C' E'  +  C D E'  +  C' D E

=  B' D' (A' C' + C E)  +  D (C E' + C' E)  +  B (D + C' E')

=  [D' + C E' + C' E] [B + D' (A' C' + C E)]  +  B (D + C' E')

=  [D' + (C + E) (C' + E')] [B + D' (A' C' + C E)]  +  B (D + C' E')

where the underlined terms use the same gate.

This uses 12 gates.

2.  Show an algebraic expression for the following circuit.  Reduce it to a minimum sum of products expression (2 terms, 4 literals.)

w  =  (a b')'  =  a' + b		y  =  c w  =  c (a b')'

x  =  c' + d				z  =  w' + x'  =  a b'  +  c d'

g  =  (y + z)'  =  (c w  +  w'  +  x')  =  (c + w' + c d')'

=  (c  +  w')'  =  (c  +  a b')'  =  c' (a' + b)

=  a' c'  +  b c'

3.  Convert the following to a sum of minterms.  Remove any duplicates from the list.

f  =  (a + b' + c) (a + c + d) (b + c') (b' + d')

=  (a + c + b' d) (b d' + b' c')

=  a b d'  +  a b' c'  +  b c d'  +  b' c' d

=  a b c' d'  +  a b c d'  +  a b' c' d'  +  a b' c' d  +  a' b c d'  +  a b c d'  			+  a' b' c' d +  a b' c' d

=  a' b' c' d +  a' b c d'  +  a b' c' d'  +  a b' c' d  +  a b c' d' +  a b c d'

4.  For the following function, find THE minimum sum of products solution.

f  =  a' b' d'  +  a b' d  +  a' b c  +  b c' d

5.  For the following function, list all of the prime implicants and find BOTH minimum sum of product expressions.  (extra maps shown for your convenience)

h (w, x, y, z)  =  ∑ m (2, 3, 5, 6, 7, 9, 10, 11, 13, 14)

w' y	   y z'*	x' y	     w x' z	   w y' z	x y' z		w' x z

h  =  y z'  +  w' y  +  x y' z  +  w x' z

h  =  y z'  +  x' y  +  w y' z  +  w' x z

6.  For the function, g, shown, show the equations and a block diagram for a minimum cost two-level NAND implementation a minimum two-level NOR implementation (spare map shown).  All variables are available both uncomplemented and complemented.

g  =  x z  +  w x'  +  w y		g'  =  w' x'  +  w' z'  +  x y' z'

g  =  (w + x) (w + z) (x' + y + z)

7.  For the function, F,  shown find a minimum sum of products expression and a minimum product of sums expression (spare map shown).

F  =  X' Z'  +  W' Y  +  W X Z		F'  =  X Z'  +  W' Y' Z  +  W X’ Z

F  =  (X' + Z) (W + Y + Z') (W’ + X + Z’)

8.  For the following function, find all 4 minimum sum of products expressions.

a b  +  {b c'  or  b d  or  c' d}		a d'  +  b d

9.  For the following 5-variable function, F, find BOTH minimum sum of product solutions.  (Two copies of the map are shown.)

F  =  A' C' D'  +  A B D  +  C D' E  +  {A' B C'  or  B C' D}

10.  For the following 5-variable function, F, find the minimum sum of product solution.  (Two copies of the map are shown.)  (6 terms, 20 literals)

F  =  A' B' C' D' E'  +  A' C D  +  A C' E  +  B C' D  +  B C D'  +  B D' E

11.  For the two functions shown, find a minimum two-level solution (map and equations) corresponding to 5 AND gates and 2 OR gates with a total of 19 inputs.  All inputs are available both uncomplemented and complemented.  (Show only the maps and equations.)

f  =  w' x'  +   w x y  +  x z

g  =  x' y' z'  +  w x y  +  x z   +  w' z

12.   For the three functions shown, find a minimum two-level solution (map and equations) corresponding to 7 AND gates and 3 OR gates with a total of 30 inputs.  All inputs are available both uncomplemented and complemented.  (Show the maps and equations.)

f  =  b' c d  +   b c d'  +  a' d  +  a c' d'

g  =  b' c' d'  +  b c' d  +  b c d'

h  =  b c' d  +  a b' d'  +  a' d