Chapter 12

12-1	Given: dmax = 25 mm, bmin = 25.03 mm, l/d = 1/2, W = 1.2 kN,  = 55 mPas, and N = 1100 rev/min.

r 25/2 = 12.5 mm

r/c = 12.5/0.015 = 833.3

N = 1100/60 = 18.33 rev/s

P = W/ (ld) = 1200/ [12.5(25)] = 3.84 N/mm2 = 3.84 MPa

Eq. (12-10):

Fig. 12-15:	h0 /c = 0.3		h0  = 0.3(0.015) = 0.0045 mm     Ans.

Fig. 12-17:	f r/c = 5.4		f = 5.4/833.3 = 0.006 48

T =f Wr = 0.006 48(1200)12.5(103) = 0.0972 Nm

Hloss = 2 TN = 2 (0.0972)18.33 = 11.2 W      Ans.

Fig. 12-18:	Q/(rcNl) = 5.1		Q = 5.1(12.5)0.015(18.33)12.5 = 219 mm3/s

Fig. 12-19:	Qs /Q = 0.81		Qs  = 0.81(219) = 177 mm3/s      Ans.

______________________________________________________________________________

12-2	Given: dmax = 32 mm, bmin = 32.05 mm, l = 64 mm, W = 1.75 kN,  = 55 mPas, and N = 900 rev/min.

r≈ 32/2 = 16 mm

r/c = 16/0.025 = 640

N = 900/60 = 15 rev/s

P = W/ (ld) = 1750/ [32(64)] = 0.854 MPa

l/d = 64/32 = 2

Eq. (12-10):

Eq. (12-20), Figs. 12-15, 12-18, and 12-20

|  | l/d | y | y1 | y1/2 | y1/4 | yl/d |
| --- | --- | --- | --- | --- | --- | --- |
| h0/c | 2 | 0.98 | 0.83 | 0.61 | 0.36 | 0.92 |
| P/pmax | 2 | 0.84 | 0.54 | 0.45 | 0.31 | 0.65 |
| Q/rcNl | 2 | 3.1 | 3.45 | 4.2 | 5.08 | 3.20 |

h0=  0.92c = 0.92(0.025) = 0.023 mm         Ans.

pmax = P / 0.65 = 0.854/0.65 = 1.31 MPa        Ans.

Q = 3.20 rcNl = 3.20(16)0.025(15)64 = 1.23 (103) mm3/s        Ans.

______________________________________________________________________________

12-3	Given: dmax = 76.2 mm, bmin = 76.327 mm, l = 38.1 mm, W = 3558.40, N = 600 rev/min,and SAE 10 and SAE 40 at 4.43C.

Fig. 12-3: SAE 10 at 4.43C,

Figs. 12-15 and 12-20: h0/c = 0.6  andP/pmax = 0.45

Fig. 12-3: SAE 40at 4.43C,

______________________________________________________________________________

12-4	Given: dmax = 82.55 mm, bmin = 82.702 mm, l = 76.2 mm, W = 3558.4 N, and N = 1000 rev/min.

Fig. 12-4: SAE 20W at 150F (65.6 C) , = 2.40 reyn =0.0196  Pa.s

Note to instructors:  Some students may obtain a higher value of viscosity (2.85) from Fig. 12-4.  The value from Fig. 12-3 is used here as the preferred value since this figure is specifically for single-viscosity oils.

From Eq. (12-20), and Figs. 12-15 and 12-20:

|  | l/d | y | y1 | y1/2 | y1/4 | yl/d |
| --- | --- | --- | --- | --- | --- | --- |
| h0/c | 0.923 | 0.82 | 0.44 | 0.26 | 0.14 | 0.42 |
| P/pmax | 0.923 | 0.83 | 0.44 | 0.31 | 0.21 | 0.42 |

Fig. 12-4: SAE 20W-40at 150F(65.6 C),  = 4.4 reyn=0.03032 Pa.s

From Eq. (12-20), and Figs. 12-15 and 12-20:

|  | l/d | y | y1 | y1/2 | y1/4 | yl/d |
| --- | --- | --- | --- | --- | --- | --- |
| h0/c | 0.923 | 0.91 | 0.6 | 0.38 | 0.2 | 0.58 |
| P/pmax | 0.923 | 0.83 | 0.48 | 0.35 | 0.24 | 0.46 |

______________________________________________________________________________

12-5	Given: dmax = 50.8 mm, bmin = 50.861 mm, l = 25.4 mm, W = 2668.8 N, N = 800 rev/min, and SAE 20 at 3.84 C.

Fig. 12-2: SAE 20at 3.84C,

From Figs. 12-15, 12-17 and 12-18:

The power loss due to friction is

______________________________________________________________________________

12-6	Given: dmax = 25 mm, bmin = 25.04 mm, l/d = 1, W = 1.25 kN,  = 50 mPas, and N = 1200 rev/min.

For µ = 50 mPa · s,

From Figs. 12-15, 12-17 and 12-19:

The power loss due to friction is

H = 2πT N = 2π(0.1125)(20) = 14.14 W Ans.

Qs= 0.57Q The side flow is 57% of QAns.

______________________________________________________________________________

12-7	Given: dmax = 31.75 mm, bmin = 31.8008 mm, l  = 50.8 mm, W = 2757.76 mm,  = 30 mPa.s, and N = 1120 rev/min.

|  | l/d | y | y1 | y1/2 | y1/4 | yl/d |
| --- | --- | --- | --- | --- | --- | --- |
| h0/c | 1.6 | 0.8 | 0.4 | 0.25 | 0.12 | 0.5 |
| fr/c | 1.6 | 2.5 | 3.2 | 4.0 | 5 | 2.6 |
| Q/rcNl | 1.6 | 2.8 | 4.3 | 5.3 | 5.8 | 3.5 |

From  Eq. (12-20), and Figs. 12-15, 12-17, and 12-1 h0 = 0.5 c = 0.5(0.0254) =0.0127 mm.          Ans.

f  = 2.6/(r/c) = 2.6/625 = 0.00416       Ans.

Q = 3.5 rcNl  = 3.5(15.785) 0.0254(18.57) 50.8 = 699.13 mm3/s      Ans.

______________________________________________________________________________

12-8	Given: dmax = 75.00 mm, bmin = 75.10 mm, l = 36 mm, W = 2 kN, N = 720 rev/min,and SAE 20 and SAE 40 at 60C.

Fig. 12-3: SAE 20 at 60C, µ = 18.5 mPa · s

From Figures 12-15, 12-17 and 12-20:

The heat loss rate equals the rate of work on the film

Hloss = 2πT N = 2π(0.51)(12) = 38.5W Ans.

pmax = 0.741/0.315 = 2.35 MPa Ans.

Fig. 12-3: SAE 40at 60C, µ = 37 mPa · s

S = 0.169(37)/18.5 = 0.338

From Figures 12-15, 12-17 and 12-20:

_____________________________________________________________________________

12-9	Given: dmax = 56.00 mm, bmin = 56.05 mm, l = 28 mm, W = 2.4 kN, N = 900 rev/min,and SAE 40 at 65C.

Fig. 12-3: SAE 40 at 65C, µ = 30 mPa · s

From Figures 12-15, 12-17, 12-18 and 12-19:

_____________________________________________________________________________

12-10	Given: SAE 40, N = 10 rev/s, Ts= 140F(60C), l/d = 1, d = 76.2 mm, b = 76.2762 mm,

W = 3002.4 N.

Trial #1: From Figure 12-3 for T = 80C, µ = 16 mPa.s,

From Fig. 12-23,

Discrepancy = 40 9.5 = 30.5°C

Trial #2: T = 65°C,µ = 29.5 mPa.s,

From Fig. 12-23,

Discrepancy = 10  16.34 = -6.34 °C

Trial #3: T = 68°C, µ = 27.5 mPa.s

From Fig. 12-23,

Discrepancy = 16  15.33 =  0.677°C    (OK)

Tav= 60 +16/2 = 68 °C   Ans.

From Figures 12-15, 12-17to 12-19:

_____________________________________________________________________________

12-11	Given: d = 63.5 mm,b = 63.6016 mm, cmin = 0.0508 mm, W= 5376 N, SAE = 20, Using Ts= 110°F , N= 1120 rev/min, and l = 63.5 mm.

P = W/(ld) = 5376/(63.5)2 = 1.33 MPa,  N = 1120/60 = 18.67 rev/s

Table 12-1:	 (reyn) = 0 (106) exp [b / (T + 95)]	b and T in F

The conversion from reyn to mPas is given in Sec. 12-2. For a temperature of C degrees Celsius,  T = 1.8 C + 32.  Substituting into the above equation gives

(mPas) = 6.89 0 (106) exp [b / (1.8 C + 32+ 95)]

= 6.89 0 (106) exp [b / (1.8 C + 127)]

For a trial film temperature, let Tf= 65°C

= (6.89)0.0136 exp[1271.6/(1.8(65) + 127)] = 17.18(10-3) Pa.s

Eq. (12-10):

Fig. 12-23:

which is not 0.1 or less, therefore try averaging for the new trial film temperature, let

Proceed with additional trialsusing a spreadsheet (table also shows the first trial)

| Trial Tf | ' | S | T | Tav | TfTav | New Tf |
| --- | --- | --- | --- | --- | --- | --- |
| 65.00 | 0.01718 | 0.0942 | 10.06 | 48.33 | 16.67 | 56.67 |
| 56.67 | 0.02417 | 0.1326 | 12.62 | 49.61 | 7.06 | 53.14 |
| 53.14 | 0.02832 | 0.1553 | 14.14 | 50.37 | 2.77 | 51.75 |
| 51.75 | 0.03021 | 0.1657 | 14.83 | 50.71 | 1.04 | 51.23 |
| 51.23 | 0.03097 | 0.1698 | 15.11 | 50.85 | 0.38 | 51.04 |
| 51.04 | 0.03125 | 0.1714 | 15.21 | 50.90 | 0.14 | 50.97 |
| 50.97 | 0.03135 | 0.1719 | 15.25 | 50.92 | 0.05 | 50.95 |

Note that the convergence begins rapidly. There are ways to speed this, but at this point

they would only add complexity.

(a)

From Fig. 12-15:

From Fig. 12-16:	= 56° Ans.

(b) e = c h0= 0.0508  0.0245 = 0.0263 mm  Ans.

(c) From Fig. 12-17:

(d) T = f Wr= 0.006 56(5376)(31.75/1000) = 1.119 N· m

(e) From Fig. 12-18:

From Fig. 12-19:

(f) From Fig. 12-20:

From Fig. 12-21:

(g) From Fig. 12-21:

(h)From the trial table,Tf= 50.95°C Ans.

(i) WithT = 15.25°C from thetrial table,Ts+ T = 43.3 + 15.25 = 48.55°C  Ans.

_____________________________________________________________________________

12-12	Given: d = 31.750 mm, td= 0.025 mm, b = 31.801 mm, tb= 0.076 mm, l = 31.75 mm, W = 1120 N, N = 1750 rev/min, SAE 10 lubricant, sump temperature Ts= 49°C.

P = W/(ld) = 1120/31.752 = 1.11 MPa,   N = 1750/60 = 29.17 rev/s

For the clearance, c = 0.051  0.025 mm. Thus, cmin = 0.025 mm, cmedian = 0.051 mm, and

cmax = 0.076 mm.

For cmin = 0.025 mm, start with a trial film temperature of Tf= 60°C

Table 12-1:	   = (6.89)0.0136 exp[1271.6/(1.8(60) + 127)] = 20.98(10-3) Pa.s

Eq. (12-10):

Fig. 12-23:

which is not 0.1 or less, therefore try averaging for the new trial film temperature, let

Proceed with additional trials using a spreadsheet (table also shows the first trial)

| Trial Tf | ' | S | T | Tav | TfTav | New Tf |
| --- | --- | --- | --- | --- | --- | --- |
| 60.00 | 0.02098 | 0.2153 | 15.15 | 56.57 | 3.43 | 58.29 |
| 58.29 | 0.02254 | 0.2314 | 16.04 | 57.02 | 1.27 | 57.65 |
| 57.65 | 0.02316 | 0.2378 | 16.40 | 57.20 | 0.46 | 57.43 |
| 57.43 | 0.02339 | 0.2401 | 16.53 | 57.26 | 0.16 | 57.34 |
| 57.34 | 0.02347 | 0.2409 | 16.57 | 57.29 | 0.06 | 57.32 |

With Tf  = 57.32°C, T = 16.57°C,  = 23.47(10-3) Pa.s,  S = 0.2409,

Tmax = Ts + T = 49 + 16.57 = 65.57°C

Fig. 12-15: 	h0/c = 0.55,   h0 = 0.55(0.025) = 0.0140 mm

= 1 h0/c = 1  0.55 = 0.45 mm

Fig. 12-17:	r f /c = 5,   f = 5/(1/625) = 0.008

Fig. 12-18:	Q/(rcNl) = 4,  Q = 4(15.875)0.0254(29.17)31.75 = 1493.6 mm3/s

Fig. 12-19:	Qs/Q = 0.54,   Qs = 0.54(1493.6) = 806.55 mm3/s

The above can be repeated for cmedian = 0.051 mm, and cmax = 0.076 mm. The results are shown below.

|  | cmin 0.025 mm | cmedian0.05 mm | cmax 0.075 mm |
| --- | --- | --- | --- |
| Tf | 57.32 | 52.68 | 51.57 |
|  | 0.0235 | 0.0289 | 0.0305 |
| S | 0.2409 | 0.0744 | 0.0346 |
|  | 16.57 | 7.29 | 5.08 |
| Tmax | 65.57 | 56.29 | 54.08 |
| h0/c | 0.55 | 0.27 | 0.15 |
| h0 | 0.01397 | 0.013716 | 0.01143 |
|  | 0.45 | 0.73 | 0.85 |
| fr/c | 5 | 2 | 1.4 |
| f | 0.008 | 0.0064 | 0.00672 |
| Q/(rcNl) | 4 | 4.6 | 4.7 |
| Q | 1493.61 | 3435.31 | 5264.98 |
| Qs /Q | 0.54 | 0.79 | 0.88 |
| Qs | 806.55 | 2713.89 | 4633.19 |

_____________________________________________________________________________

12-13	Computer programs will vary.

_____________________________________________________________________________

12-14	In a step-by-step fashion, we are building a skill for natural circulation bearings.

• Given the average film temperature, establish the bearing properties.

• Given a sump temperature, find the average film temperature, then, establish the

bearingproperties.

• Now we acknowledge the environmental temperature’s role in establishing the sump

temperature. Sec. 12-9 and Ex. 12-5 address this problem.

Given: dmax = 63.5 mm, bmin = 63.605,mm l/d = 1, N = 1120 rev/min, SAE 20 lubricant, W = 1344 N, A = 38700 mm2, Using T = 21C, and  = 1.

2688 N load with minimal clearance: We will start by using W = 2688 N (nd = 2).The 	task is to iteratively find the average film temperature, Tf, which makes Hgen and Hloss	equal.

N = 1120/60 = 18.67 rev/s

Table 12-1:	  = (6.89)0.0136 exp[1271.6/(1.8  + 127)]

Start with trial values of Tf of 105 and 115C.

| Trial Tf |  | S | f r/c | Hgen | Hloss |
| --- | --- | --- | --- | --- | --- |
| 105 | 0.00524 | 0.0553 | 1.8 | 28.51 | 24.87 |
| 115 | 0.00422 | 0.0445 | 1.6 | 25.34 | 27.83 |

As a linear approximation, let Hgen = mTf  +b. Substituting the two sets of values of Tf	and Hgen we find that Hgen =  0.317 Tf  +61.8. Setting this equal to Hloss and solving 	for Tf  givesTf  = 111C.

| Trial Tf |  | S | f r/c | Hgen | Hloss |
| --- | --- | --- | --- | --- | --- |
| 111 | 0.00459 | 0.0484 | 1.7 | 26.9 | 26.6 |

which is satisfactory.

Fig. 12-15:	h0/c = 0.21,      h0 = 0.21 (0.05) = 0.0105 mm

Fig. 12-23:

T1 = Ts = TfT = 111  3.69/2 = 109.2C

Tmax = T1 + T = 109.2 + 3.69 = 112.9C

Trumpler’s design criteria:

0.00508 + 0.000 04d = 0.002 + 0.000 04(63.5) = 0.0076 mm <h0O.K.

Tmax = 112.9FC< 121C   O.K.

nd  =  2  (assessed at W = 2688 N)     O.K.

We see that the design passes Trumpler’s criteria and is deemed acceptable.

For an operating load of W = 1344 N, it can be shown that Tf  = 104.7C,  = 0.00528, S = 0.111, f r/c =  3.09, Hgen = Hloss = 23.75 W, h0 =0.019 , T = 2.9C, T1 = 103.2C, andTmax = 106.1C.

_____________________________________________________________________________

12-15	Given: , SAE 30, Using Ts = 49C, ps = 0.345 MPa, N = 2000/60 = 33.33 rev/s, W = 20.7 kN, bearing length = 50.8 mm, groove width = 6.35 mm, and Hloss 1.46 kW.

r=d/ 2 = 88.9/2 = 44.45 mm

r / c = 44.45/0.06 = 700

l  = (50.8 6.35)/2 = 22.25  mm

l  / d = 22.25/88.9 = 0.25

Trial #1: Choose (Tf)1 = 65°C. From Table 12-1,

= (6.89)0.0136 exp[1271.6/(1.8(65)  + 127)] = 0.0256 Pa.s

From Figs. 12-15 and 12-17:	= 0.9,   f r/ c =3.6

From Eq. (12-31),

Tav = Ts  +T / 2 = 49 + 41.7/2 = 69.9C

Trial #2: Choose (Tf)2 = 71°C. From Table 12-1

= (6.89)0.0136 exp[1271.6/(1.8(71)  + 127)] = 0.0202 Pa.s

From Figs. 12-15 and12-17:	= 0.91,  f r/ c =3

Tav  =49 + 26.2/2 = 62.1C

Trial #3: Thus, the plot gives (Tf)3 = 66.9°C. From Table 12-1

= (6.89)0.0136 exp[1271.6/(1.8(66.9)  + 127)] = 0.0237 Pa.s

From Figs. 12-15 and12-17:= 0.91,  f r/ c =3.2

Tav  =69 + 33.88/2 = 65.9C

Result is close. Choose

= (6.89)0.0136 exp[1271.6/(1.8(66.4)  + 127)] = 0.0242 Pa.s

h0 = 0.095(0.0635) = 0.0059 mm

Tmax = Ts + T = 49 + 34.38 =83.4C

Eq. (12-28):

Hloss = CpQsT = 861(1758)16245(10-9)(34.8) = 855 W    O.K.

= 0.877(602) = 3160 Btu/h     O.K.

Trumpler’s design criteria:

0.00508 + 0.000 04(87.5) = 0.00858 mm > 0.00594      Not O.K.

Tmax= 83.8°C < 121°C    O.K.

Pst = 5.24 MPa > 2.068 MPa    Not O.K.

n = 1,    as done    Not O.K.

_____________________________________________________________________________

12-16	Given: , SAE 30, Ts = 55C, ps = 200 kPa,

N = 2880/60 =48 rev/s, W = 10 kN, bearing length = 55 mm, groove width = 5 mm, and Hloss 300 W.

r =d/ 2 = 50/2 = 25 mm

r/c = 25/0.042 = 595

l  = (55  5)/2 = 25 mm

l  / d = 25/50 = 0.5

Trial #1: Choose (Tf)1 = 79°C. From Fig. 12-3, µ = 13 mPa · s.

From Figs. 12-15 and 12-17:= 0.85,   f r/ c = 2.3

From Eq. (12-32),

Tav = Ts  +T / 2 = 55 + 76.3/2 = 93.2C

Trial #2: Choose (Tf)2 = 100°C. From Fig. 12-3, µ = 7 mPa · s.

From Figs. 12-15 and12-17:= 0.90,  f r/ c =1.6

Tav  = 55 + 26.9/2 = 68.5C

Trial #3: Thus, the plot gives (Tf)3 = 85.5°C. From Fig. 12-3, µ = 10.5 mPa · s.

From Figs. 12-15 and12-17:= 0.87,  f r/ c =2.2

Tav  = 55 + 58.9/2 = 84.5C

Result is close. Choose

Fig. 12-3: µ = 10.5 mPa · s

From Eq. (12-28)

h0 = 0.13(0.042) = 0.005 46 mm or 0.000 215 in

Tmax = Ts + T = 55 + 58.9 = 113.9C     or     237°F

Hloss = CpQsT = 0.0311(0.42)0.193(138) = 0.348 Btu/s

= 1.05(0.348) = 0.365 kW = 365 W not O.K.

Trumpler’s design criteria:

0.0002 + 0.000 04(50/25.4) = 0.000 279 in>h0Not O.K.

Tmax=  113.9 C     or  237°F O.K.

Pst = 4000 kPa    or    581 psi> 300 psi Not O.K.

n = 1,    as done    Not O.K.

_____________________________________________________________________________

12-17	So far, we have performed elements of the design task. Now let’s do it more completely.

The values of the unilateral tolerances, tband td, reflect the routine capabilities of thebushing vendor and the in-house capabilities. While the designer has to live with these,his approach should not depend on them. They can be incorporated later.

First we shall find the minimum size of the journal which satisfies Trumpler’s constraintof Pst≤ 2.07 MPa.

In this problem we will take journal diameter as the nominal value and the bushing boreas a variable. In the next problem, we will take the bushing bore as nominal and the journaldiameter as free.

To determine where the constraints are, we will set tb= td = 0, and thereby shrinkthe design window to a point.

We set 			d = 50.0 mm

b = d + 2cmin = d + 2c

nd= 2 (This makes Trumpler’snd≤ 2 tight)

and construct a table.

| c | b | d |  | Tmax | h0 | Pst | Tmax | n | fom |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.025 | 50.05 | 50 | 101 | 153 |  |  |  |  | 5.74 |
| 0.0275 | 50.055 | 50 | 96.9 | 144 |  |  |  |  | 6.06 |
| 0.03 | 50.06 | 50 | 93 | 136 |  |  |  |  | 6.37 |
| 0.0325 | 50.065 | 50 | 89.7 | 128.8 |  |  |  |  | 6.66 |
| 0.035 | 50.07 | 50 | 86.6 | 122.8 |  |  |  |  | 6.94 |
| 0.0375 | 50.075 | 50 | 84.3 | 117.5 |  |  |  |  | 7.20 |
| 0.04 | 50.08 | 50 | 82 | 112.8 |  |  |  |  | 7.45 |
| 0.0425 | 50.085 | 50 | 80.2 | 108 |  |  |  |  | 7.65 |
| 0.045 | 50.09 | 50 | 78.2 | 104.9 |  |  |  |  | 7.91 |
| 0.0475 | 50.095 | 50 | 76.6 | 101.5 |  |  |  |  | 8.12 |
| 0.05 | 50.1 | 50 | 75.2 | 98.4 |  |  |  |  | 8.32 |

*Sample calculation for the first entry of this column.

Iteration yields:

With , from Table 12-1

From Figs. 12-15 and 12-17:	= 0.7, f r/c = 5.5

Eq. (12–31):

For the nominal 50 mm bearing, the various clearances show that we have been in contactwith the recurving of (h0)min.The figure of merit (the parasitic friction torque plus thepumping torque negated) is best at c = 0.045 mm. For the nominal 50 mm bearing, we will place the top of the design window at cmin = 0.05 mm, and b = d + 2(0.05) = 50.1 mm. At this point, add the b and d unilateral tolerances:

Now we can check the performance at cmin ,, and cmax . Of immediate interest is the fomof the median clearance assembly, 9.82, as compared to any other satisfactory bearingensemble.

If a nominal 47.62 mm bearing is possible, construct another table with tb= 0 andtd= 0.

| c | b | d |  | Tmax | h0 | Pst | Tmax | n | fom |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.05 | 46.9 | 46.8 | 73.7 | 91.1 |  |  |  |  | 7.36 |
| 0.075 | 46.95 | 46.8 | 65 | 73.7 |  |  |  |  | 8.64 |
| 0.0875 | 46.975 | 46.8 | 62.6 | 69 |  |  |  |  | 9.05 |
| 0.1 | 47 | 46.8 | 60.9 | 65.7 |  |  |  |  | 9.32 |
| 0.125 | 47.05 | 46.8 | 58.9 | 61.6 |  |  |  |  | 9.59 |
| 0.1375 | 47.075 | 46.8 | 58.3 | 60.4 |  |  |  |  | 9.63 |
| 0.15 | 47.1 | 46.8 | 57.8 | 59.5 |  |  |  |  | 9.64 |

The range of clearance is 0.075 < c <0.1375 mm. That is enough room to fit in our design window.

The ensemble median assembly has a fom = 9.31.

We just had room to fit in a design window based upon the (h0)min constraint. Further reduction in nominal diameter will preclude any smaller bearings .A table constructed for a d = 44 mm journal will prove this.

We choose the nominal 46.8 mm bearing ensemble because it has the largest figure of merit. Ans.

_____________________________________________________________________________

12-18	This is the same as Prob. 12-17 but uses design variables of nominal bushing bore b andradial clearance c.

The approach is similar to that of Prob. 12-17 and the tables will change slightly. In the table for a nominal b = 47.6 mm, note that at c = 0.075 mm the constraints are “loose.” Set

b = 47.6 mm

d = 47.6  2(0.075) = 47.47 mm

For the ensemble

Analyze at cmin = 0.075 mm, = 0.1 mm and cmax = 0.125 mm

At and theTrumpler conditions are met.

At   = 0.02668 Pa.s, S = 0.0205, Hloss = 320 W, fom = 9.246 and the Trumpler conditions are O.K.

At   = 0.0298 Pa., S = 0.014 66, Hloss = 326 W and the Trumpler conditions are O.K.

The ensemble figure of merit is slightly better; this bearing is slightly smaller. The lubricantcooler has sufficient capacity.

_____________________________________________________________________________

12-19	Table 12-1:	 (reyn) = 0 (106) exp [b / (T + 95)]	b and T in F

The conversion from reyn to mPas is given in Sec. 12-2. For a temperature of C degrees Celsius,  T = 1.8 C + 32.  Substituting into the above equation gives

(mPas) = 6.89 0 (106) exp [b / (1.8 C + 32+ 95)]

= 6.89 0 (106) exp [b / (1.8 C + 127)]   Ans.

For SAE 50 oil at 70C, from Table 12-1, 0 = 0.0170 (106) reyn, and b = 1509.6F. From the equation,

= 6.89(0.0170)106(106) exp {1509.6/[1.8(70) + 127]}

= 45.7 mPas              Ans.

From Fig. 12-3,    = 39 mPas              Ans.

The figure gives a value of about 15 % lower than the equation.

_____________________________________________________________________________

12-20	Originally

Doubled,

The radial load quadrupled to 16 kN when the analyses for parts (a) and (b) were carriedout. Some of the results are:

| Part |  |  | S |  | f r/c | Qs | h0 /c |  | Hloss | h0 | Trumpler h0 | f |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (a) | 0.177 | 0.0235 | 0.0310 | 75.1 | 0.1612 | 6.56 | 0.1032 | 0.157 | 10393 | 0.01806 | 0.009 | 0.005 67 |
| (b) | 0.089 | 0.0235 | 0.0310 | 75.1 | 0.1612 | 0.870 | 0.1032 | 0.157 | 1299 | 0.00903 | 0.007 | 0.005 67 |

The side flow Qsdiffers because there is a c3 term and consequently an 8-fold increase.

Hloss is related by a 9898/1237 or an 8-fold increase. The existing h0is related by a 2-foldincrease. Trumpler’s (h0)min is related by a 1.286-fold increase.

See spreadsheets for Problems 12-21, 12-22, and 12-23

_____________________________________________________________________________

12-21	(a) Δt = (0.5 deg)(1 s / 200 rad)(π rad / 180 deg) = 43.633(10-6) s

c = 25(10-6) m

l = 0.025 m

d = 0.050 m

μ = 7(10-3) N-s/m2

Using the procedure outlined in Example 12-7, the results after two Euler time steps are

t = 0 s			t = 43.633(10-6) s		t = 87.266(10-6) s

(0 deg)			(0.5 deg)			(1.0 deg)

Fx (N)		3000			3000				3000

Fy (N)		0			   0				   0

F (N)		3000			3000				3000

cos		   1			   1				   1

sin 		0			   0				   0

ε		0.8			0.713100			0.636471

ε		0.3			0.285328			0.272453

M0		1.430331		1.259763			1.120271

M		5.721322		5.039051			4.481084

M		1.214102		1.068612			0.949896

Mx		5.721322		5.039051			4.481084

My		1.214102		1.068612			0.949896

100			100				100

F(c/r)2/

(μld/c)	8.571429(10-3)		8.571429(10-3)			8.571429(10-3)

vx (m/s)	4.978990(10-2)		4.390518(10-2)			3.909042(10-2)

vy (m/s)	8.406591(10-3)		7.376781(10-3)			6.550789(10-3)

εx		0.8			0.713100			0.636471

εy		0.3			0.285328			0.272453

Plotting eccentricity ratio components at 720 successive Euler time steps from t = 0 s

(0 deg) to t = 3.1416(10-2) s (360 deg) gives the following journal orbit:

Ans.

Values of journal eccentricity ratio at selected times:

time (deg)	εxεy	ε

0.0    		0.8000     0.3000 	0.8544

90.0    		 0.6498      0.2020 	0.6805

180.0    		 0.6293      0.4211 	0.7572			Ans.

270.0     	0.5983      0.4453 	0.7458

360.0   		 0.5964      0.4362	0.7389

(b)		N = (200 rad/s) (1 rev/2π rad) = 100/π rev/s

Eq. (12-10):  S = (r/c)2 (μN/P) = 0.0928

Fig. 12-15: h0 / c = 0.2      h0 = 0.2(25)(106) =  5(106) = 5 m     Ans.

Fig. 12-16:l/d = 0.025/0.05 = 1/2      	  = 330		Ans.

From part (a), ε = (εx2 + εy2)1/2 = [(0.5964)2 + (0.4342)2]1/2 = 0.74

h0 = c (1 ε) = 25(106) (1  0.74) = 6.5 (106) m = 6.5 m     Ans.

= atan (εy / εx ) = atan (0.4342 / 0.5964) = 36.10	Ans.

The answers for (a) and (b) are somewhat different because film thickness and attitude angle predicted by Raimondi and Boyd is a numerical solution of the complete Reynolds equation, while the short bearing solution neglects circumferential flow.  In addition, a small amount of error is contained in the curve fitted mobility expressions of Eq. 12-35.      Ans.

12-22	(a)	Δt = (0.5 deg)(1 s / 200 rad)(π rad / 180 deg) = 43.633(10-6) s

c = 25(10-6) m

l = 0.025 m

d = 0.050 m

μ = 7(10-3) N-s/m2

Using the procedure outlined in Example 12-7, the results after two Euler time steps are

t = 0 s			t = 43.633(10-6) s	t = 87.266(10-6) s

(0 deg)			(0.5 deg)			(1.0 deg)

Fx (N)		3000			2999.886			2999.543

Fy (N)		0			26.17961			52.35722

F (N)			3000			3000				3000

cos 			1			0.999962			0.999848

sin 			  0			0.008727			0.017452

ε				0.8			0.715563			0.640985

ε				0.3			0.279094			0.260960

M0				1.430331		1.264414			1.128209

M				5.721322		5.057655			4.512834

M				1.214102		1.047618			0.913754

Mx				5.721322		5.048321			4.496200

My				1.214102		1.091714			0.992375

(rad/s)		100			100				100

F(c/r)2/

(μld/c)		8.571429(10-3)		8.571429(10-3)			8.571429(10-3)

vx m/s)		4.978990(10-2)		4.398464(10-2)			3.921912(10-2)

vy (m/s)		8.406591(10-3)		7.574801(10-3)			6.915238(10-3)

εx				0.8			0.713100			0.636471

εy				0.3			0.285328			0.272453

Plotting eccentricity ratio components at 720 successive Euler time steps from t = 0 s (0 deg) to t = 3.1416(10-2) s (360 deg) gives the following journal orbit:

Ans.

Values of journal eccentricity ratio at selected times:

time (deg)	εx		εy	ε

0.0    		0.8000    	0.3000     		 0.8544

90.0     		0.2507      	0.6240      		0.6725

180.0    		0.6222     	 0.4211      		0.7513

270.0   		0.4456     	0.5995      		0.7469

360.0     		0.5972     	0.4399      		0.7418 	Ans.

(b) The periodic orbit is approaching a circle of radius 0.74.  Cyclic minimum film 	thickness is identical to that in part (a).  If an observer is attached to the moving 	load, the load and journal appear to be non-rotating, while the sleeve appears to rotate at a constant angular velocity of −200 rad/s (clockwise).    Ans.

12-23	(a)	Δt = (0.5 deg) (1 s / 200 rad) (π rad / 180 deg) = 43.633(10-6) s

c = 25(10-6) m

l = 0.025 m

d = 0.050 m

μ = 7(10-3) N-s/m2

Using the procedure in Example 12-7, the results after two Euler time steps are

t = 0 s		t = 43.633(10-6) s		t = 87.266(10-6) s

(0 deg)			(0.5 deg)			(1.0 deg)

Fx (N)		3000			2999.971			2999.886

Fy (N)		   0			13.08993			26.179606

F (N)		3000			3000				3000

cos 			1			0.999990			0.999862

sin 			   0			0.004363			0.008727

ε				0.8			0.714338			0.638753

ε				0.3			0.282214			0.266716

M0				1.430331		1.262100			1.124280

M				5.721322		5.048399			4.497118

M				1.214102		1.058145			0.931923

Mx				5.721322		5.043734			4.488815

My				1.214102		1.080162			0.971132

(rad/s)		100			100				100

F(c/r)2/

(μld/c)		8.571429(10-3)		8.571429(10-3)		8.571429(10-3)

vx m/s)		4.978990(10-2)		4.394533(10-2)		3.915625(10-2)

vy m/s)		8.406591(10-3)		7.475784(10-3)		6.732986(10-3)

εx				0.8			0.713100			0.636401

εy				0.3			0.285328			0.272280

Plotting eccentricity ratio components at 2880 successive Euler time steps from t = 0 s (0 deg) to t = 0.125664 s (1440 deg) gives the following journal orbit

Ans.

Values of journal eccentricity ratio at selected times:

time (deg)	εx		εy	ε

0.0    	0.8000    	0.3000      	0.8544

360.0    	0.8679      0.0123      	0.8679

720.0     0.9191     0.0073     	0.9191

1080.0   	0.9417      0.0056     	0.9417

1440.0     0.9559     0.0047      	0.9559 Ans.

(b) The minimum film thickness is slowly approaching zero.  If an observer is attached 	to the moving load, thejournal appears to have a counterclockwise angular velocity 	of +100 rad/s, while the sleeve appears to have a clockwise angular velocity of 	−100 rad/s.  The average angular velocity of journal and sleeve is zero, resulting in 	pure squeeze motion from the viewpoint of the load.   Ans.

12-24	Since the load is identically zero, the first term on the right hand side of Eq. (12-34) is zero, so that

__

de/dt =   ω k  xe							(a)

With c = 25(10-6) m, the eccentricity vector and its magnitude at equilibrium as found from Problem 12-21 is

e  =cε  = 25(106)(0.5964 i + 0.4362 j)

= 14.91(106) i + 10.91(106) j   m

e = e = {[14.91(106)]2 + [10.91(106)]2}1/2  =  18.47 (106)  m

= 18.47 μm

According to Eq. (a), the instantaneous velocity vector of the journal center is always 90 degrees to the journal eccentricity vector. Therefore, the journal center moves along a circle of radius 18.47 μm. The journal orbit angular velocity is ½ the journal angular velocity.     Ans.

12-25	Sample calculation for engine speed N = 3000 rev/min:

From Ex. 12-8,   Pcyl = 7(106) N/m2, Acyl = 0.0043 m2, Mrec = 0.64 kg, Mrot = 0.41 kg,

R = 0.0864/2 = 0.0432 m (1/2 stroke), c = 20(10-6) m, μ = 0.004332 N-s/m2, l = 0.0253 m, 	d = 0.0506 m.

= 2 N/ 60 = 2  (3000)/60 = 314.16 rad/s

8MrotRωc2/[μl3d] =

{ 8(0.41)(0.0432)(314.16)[(20)(10-6)]2 } / [(0.004332)(0.02533)(0.0506)] =

5.0

Mrec/Mrot = 0.64/0.41 = 1.56

From Fig. 12-38,hmin/c ≈ 0.23, so hmin ≈ (0.23)[20(10-6)] = 4.6(10-6) m =  4.6μm.

Repeat the calculation for N in the range 500 – 8000 rev/min and plotting gives

Ans.

Checking applicability of the chart using Eqs. (12-36) and (12-37)

Repeat the calculation for N in the range 500 – 8000 rev/min and plotting gives

Ans.

From Eq. (12-36), applicability is based on PcylAcyl 7 Frot. Thus, the chart is applicable for engine speeds > 4700 rev/min.

Over the range of applicability, the film thickness is above the danger level. Therefore, the design is acceptable for engine speeds in the range 4700 to 8000 rev/min. No claim to design acceptability is possible from the chart for speeds below 4700 rpm.Ans.

______________________________________________________________________________

12-26	Given: Oiles SP 500 alloy brass bushing, L = 19.05 mm, D = 19.05 mm, Using T = 21C, F = 1798 N, N = 250 rev/min, and w = 0.101 mm.  ,

Table 12-9:	PVmax = 1.64 MPa*m/s,   Pmax = 24.5 MPa,   Vmax = 0.51 m/s

PV = 4.95 (0.25) = 1.23 MPa*m/s  < 1.64 MPa*m/s   O.K.

Table 12-9:	K = 1207(1010) m3s/(Nms)

P = F/ (DL) = 1798/ [19.05(19.05)] = 4.95 MPa

V = DN =  (0.01905)4.17 = 0.25 m/s

Eq. (12-43) can be written as

Solving for t,

Cycles = Nt = 250 (68012) = 17.003 (106) cycles           Ans.

_____________________________________________________________________________

12-27	Given: Oiles SP 500 alloy brass bushing, wmax = 0.05 mm for 1000 h, N = 200 rev/min,

F = 448 N, CR = 15.3 W/(m2∙oC), Tmax = 149 oC,  fs = 0.03, and nd = 2.

Using Eq. (12-49) with ndF for F, fs = 0.03 from Table 12-10, and CR = 15.3

W/(m2C), gives

From Table 12-12, the smallest available bushing has an ID = 25.4  mm, OD = 34.92 mm, and L = 50.8 mm. With L/D = 50.8/25.4 = 2, this is inside of the recommendations of Eq. (12-44). Thus, for the first trial, try the bushing with ID = 25.4 mm, OD =34.92 mm, and L = 50.8 mm. Thus,

Eq. (12-42):

Eq. (12-40):

PV = 0.69(265.67) = 36.13 MPa mm/s <  1640 MPa mm/s    (OK)

Eq. (12-43), with Table 12-9:

w  =KndFNt /(3L)

= 1207(10-20)(2)(448)(200/60)(1000)(3600)/[(3)(0.0508)] = 0.000851 mm < 0.0254 mm (OK)

Answer	Select ID = 25.4 mm, OD =34.92 mm , and  L = 50.8 mm.